AMB Stuttgart

13 – 17 September2016

Hall 1 | Stand 1I16

The advanced Supermini: Frequently imitated but never matched

Some new high-performance versions of Horn’s 105 Supermini are set to be unveiled at AMB 2016. Featuring a new coating, a new substrate and a new microgeometry, this equipment will set new standards for boring out holes of between 0.2 mm (0.0079") and 6.8 mm (0.2677"). By introducing this equipment, Horn are responding to the wishes of its customers who are having to contend with higher and higher percentages of stainless/high-alloy/inhomogeneous steels and want to accelerate their throughput. Suitable for hole diameters of between 0.2 mm (0.0079") and 6.8 mm (0.2677"), the 105 Supermini tool system offers more than 1500 cutting insert versions for many different machining tasks. It truly shines in a whole range of applications for boring out, grooving, chamfering, part-off reparation threading, axial grooving, finish-boring, face turning and broaching right down to the smallest of diameters. Its existing range of uses (machining of steel, cast iron, non-ferrous metals, hard and exotic alloy materials and machining of small and miniature parts with a special microgeometry) is now being extended by introducing high-performance versions for faster machining of demanding steel grades such as stainless and high-alloy steels as well as those that are inhomogeneous to varying degrees.

Highly homogeneous tough substrate

To further improve the performance of the highly advanced 105 system, various adjustments had to be made one after another in a logical order: first to the substrate, then to the microgeometry and finally to the coating. During the first few test series, only the substrate was modified, with all the other parameters remaining unchanged. As part of the optimisation process, we were able to improve the performance by an average of 30 per cent simply by modifying the substrate. Nevertheless, it proved extremely difficult to create a highly homogeneous substrate because of the narrow process window.

Improved cutting ability combined with lower cutting pressure

In addition, we investigated the relationship between the microgeometry and the coating with regard to layer adhesion and residual stress. The tests revealed that although the modified microgeometry resulted in higher levels of cutting edge stress, the tougher substrate was able to absorb this. The sharper cutting edge geometry resulted in improved cutting ability and less cutting pressure – although a higher level of stress was exerted on the cutting edge. To counteract this higher stress, the coating specialists at Horn used a specially prepared substrate as the starting point for developing a coating system that was tailored to the combination of the new tough substrate plus more powerful cutting edge. They developed a completely new technology for depositing a denser layer with a more homogeneous structure. This new smoother layer improved the anti-friction properties significantly. Less friction means that less heat is transferred into the tool and, in turn, that the direct cutting edge is exposed to less thermal stress. The layer is topped with a golden top layer coat so that wear can be detected more easily. This complex optimisation process clearly revealed the relationship between the following factors: layer adhesion on the substrate, the layer structure, and the residual stresses. The adhesion of the first layer to the substrate by means of mechanical interlocking turned out to be crucial for ensuring a good balance between layer adhesion and residual stress.

Significant increase in tool life

Comparisons with existing substrates and coatings were carried out in the context of extensive trials and these demonstrated that the new solution resulted improving in a tool life that on existing data. Once secure in this knowledge, Horn made the “high-performance Supermini” available to customers for testing. The documented results of customer trials show that the new EG35 grade is improved: when used on Co28Cr6Mo0,2C with a strength of 1000 N/mm², the tool life was 60 per cent better. In the case of 11SMN30+C, the tool life increased by more than 100 per cent and with 40CrMoV13-9 the increase was even greater compared to competitors inserts.

Launch scheduled for the AMB in Stuttgart

The new high-performance version of the Supermini for boring out featuring the new substrate, new microgeometry and new coating will be available as of AMB 2016. The new 105 type really stands out thanks to the brand-new TiAlN coating with its golden top layer for detecting wear, excellent hardness, extremely dense layer structure and superb layer adhesion. The in-house coating process makes for short delivery times and even enables “GreenLine” processing. The Supermini is available with diameters ranging from 0.2 mm (0.0079") to 6 mm (0.2362") as standard. The standard lengths range from 5 x D for a diameter of 0.2 mm (0.0079") and up to 35 mm (1.378") for a diameter of 6 mm (0.2362"). As with all the other versions of the Supermini, the high-performance cutting inserts – come in a right or left-hand versions – fit into the tried-and-tested holder system that is available with or without internal cooling.

5.197 characters incl. spaces

Image caption:

Image 1: Everything is new: New substrate, new microgeometry, new coating.

Image 2: The Supermini – for bore machining.

Image 3: The Supermini is one of Horn’s core products and core competencies.

Image credits: Paul Horn GmbH, Nico Sauermann

Text and further information:

Hartmetall-Werkzeugfabrik Paul Horn GmbH, Mr Christian Thiele

Unter dem Holz 33-35, 72072 Tübingen, Germany

Tel.: +49 (0) 7071 7004-1602, Fax: +49 (0) 7071 72893

E-mail: christian.thiele@phorn.de, www.phorn.de