Intec, Leipzig

Hall 3 / Stand F04/G05

Hall 2 / Stand B01

Triple-edged grooving insert with central screw

Our new 34T triple edged grooving Insert is an excellent addition to the Horn range and in some applications the advantages of the 34T insert can be offered as an alternative to our existing and highly acclaimed, 312 grooving system.

The precision-sintered grooving insert can be used as a neutral insert and as both a left-hand and a right-hand insert. Furthermore, no additional clamping elements, which may have a negative effect on chip flow, are required. During clamping, the cutting insert lines up precisely with the top of the holder. In the insert seat, the insert lies flat on three sides, thereby lending the overall system a high degree of stability and also maximising repeat accuracy.

The grooving insert is centrally screwed into the insert seat using a clamping bolt, creating a precise and reliable connection. For this purpose, and despite the narrow ridge width, a counterbore has been added on both sides of the insert. This is made possible thanks to state-of-the-art pressing technologies in the blank manufacturing sector which have only recently been introduced at Horn. The screw head that is countersunk as a result does not produce interfering contours and enables reliable grooving even on a collar or directly on the spindle.

The system allows free chip flow at grooving depths of up to 6.5 mm and grooving widths of 2 to 5 mm; the geometric forms .3, .5 and .D that are currently available are among the features that ensure this. All of these features combined mean that the equipment can be used in a wide range of applications including grooving, longitudinal turning and copying of shaped grooves even for materials of medium and higher strengths. At the AMB, matching square shank holders will be available in left-hand and right-hand design with dimensions of 16 x 16, 20 x 20 and 25 x 25 mm. The cutting inserts in 2 and 2.5 mm cutting widths fit into insert seat size 1 in the same holder. The other insert seats have been adapted to the corresponding cutting widths in millimetre increments. All holders are equipped with internal cooling. Coolant is supplied to the cutting edge from the side through multiple coolant outlets as well as to the cutting point and free-formed surface of the insert from below. This method of cooling makes for long tool lives as well as ensuring that chips are flushed away. The coolant is transferred via a slot at the bottom of the holder so that a variety of unclamping lengths can be accommodated. The holders have been designed for compatibility with the relevant VDI supports.

2,600 characters incl. spaces

Image captions:

Image 1: Screwed grooving insert type 34T

Image2: Details of the screwed grooving insert type 34T

Image credits: Paul Horn GmbH, Nico Sauermann

Text and further information:

Hartmetall-Werkzeugfabrik Paul Horn GmbH, Mr Christian Thiele

Unter dem Holz 33-35, 72072 Tübingen, Germany

Tel.: +49 7071 7004-1602, Fax: +49 7071 72893

E-mail: christian.thiele@phorn.de, www.phorn.de