Intec, Leipzig

Hall 3 / Stand F04/G05

Hall 2 / Stand B01

Milling system with patented tangentially screwed indexable inserts

The patented HORN tangential milling system has been extended to include another milling range. The new 406 milling system is designed as an end-milling range for smaller cutting edge diameters. Type 406 milling inserts expand the 409 tangential milling system downwards. The 406 system is equipped with a four-edged rhombic indexable insert. The milling shanks with DIN 1835-B holders are available to offer cutting edge diameters of 16, 20, 25, 32 and 40 mm, and come equipped with two to six 406-type cutting inserts. The precision-ground rhombic indexable inserts achieve a high level of precision with very good surface quality levels. Positive cutting and axial angles enable a soft cut. The secondary cutting edge with integrated trailing chamfer produces outstanding surface finish. An additional free-formed surface chamfer provides a stable wedge angle and a very smooth milling process. The indexable inserts in carbide grade AS4B, a tough basic substrate, and are coated with TiAlN.

Chip flow has been optimised thanks to a cutting edge geometry developed using chip shape simulation. "Projections" attached to the cutting inserts also produce an improved relief angle for the individual secondary cutters and, furthermore, provide extra protection on the flat side for the main cutting edge, which is set back in this case. Cutting inserts with a corner radius of 0.4 mm achieve cutting depths of up to 6.3 mm and are suitable for milling exact 90° shoulders. The entire cutting length can be used during this process.

The machining process is assisted by effective internal cooling through the holder with cooling lubricant outlet in the direction of the cutting zone. Furthermore, the special surface treatment of the milling cutter body provides protection against abrasive chips by affording hardness and rigidity.

On trial,the advantages were clear to see in the machining of tempered material. At a cutting speed of 180 m/min, the type 406 milling cutters in 16 mm diameter were measured to exert approx. 25% less stress than comparable competitor tools which are also equipped with tangentially screwed indexable inserts.

2,206 characters incl. spaces

Image captions:

Image 1: The milling cutters are equipped with rhombic tangentially screwed indexable inserts

Image 2: 406 end mills for cutting depths of up to 6.3 mm

Image credits: Paul Horn GmbH, Nico Sauermann

Text and further information:

Hartmetall-Werkzeugfabrik Paul Horn GmbH, Mr Christian Thiele

Unter dem Holz 33-35, 72072 Tübingen, Germany

Tel.: +49 7071 7004-1602, Fax: +49 7071 72893

E-mail: christian.thiele@phorn.de, www.phorn.de