Intec, Leipzig

Hall 3 / Stand F04/G05

Hall 2 / Stand B01

DCX solid carbide slot millers with expanded dimensions

The DCX solid carbide slot milling system has been developed as an addition to the circular milling system with exchangeable inserts and 3 or 6 cutting edges. It comes with the advantage of increased slot depths when using cutting edge diameters of 20 mm (0.7874"), 25 mm (0.9843"), 30 mm (1.1811"), 35 mm (1.3780") and 40 mm (1.5748").

In the future, the product family will add longer shaft lengths and wider profile widths manufactured according to customer specifications. HORN developed the DCX solid carbide millers especially for circular milling of deep and narrow slots in steel and stainless steels. One of their areas of application is manufacturing surgical instruments and forceps. HORN grade AS45 is used as the cutting material. It is characterised by a long service life and low cutting forces when milling steel.

The exceptional product characteristics of these DCX solid carbide millers include:

–	Increased slot depths of up to 15 mm (0.5906") for D = 40 mm (1.5748")

–	Large chip areas for preventing chip blockage

–	HORN-specific cutting geometry for easy cutting

–	Adjusted number of teeth for high cutting performance with low power consumption

–	High-performance TiAlN coating for slot milling applications

As well as this, additional customer-specific solutions are available with the following maximum dimensions:

–	Shaft length of up to 140 mm (5.5118") for large overhangs

–	Profile width of up to 10 mm (0.3937") and slot depth of up to 15 mm (0.5906") for D = 40 mm (1.5748")

–	Profile width of up to 8 mm (0.3150") for D = 25 mm (0.9843")

– Tool design according to customer requirements where diameter, profile shape, profile width and shaft length are concerned.

1,739 characters incl. spaces

Image caption:

Image 1: DCX with longer shaft lengths and profile width for special applications

Image credit: Paul Horn GmbH, Nico Sauermann

Text and further information:

Hartmetall-Werkzeugfabrik Paul Horn GmbH, Mr Christian Thiele

Unter dem Holz 33-35, 72072 Tübingen, Germany

Tel.: +49 7071 7004-1602, Fax: +49 7071 72893

E-mail: christian.thiele@phorn.de, www.phorn.de